REGISTER AND ITS TYPES

What is a Register?

A register is a very small and very fast memory location inside the CPU (Central Processing Unit).

It is used to:

- Store data temporarily

- Store instructions

- Store memory addresses

- Perform calculations quickly

Registers are faster than RAM because they are inside the processor.

Example: When you add two numbers, the CPU stores them in registers first, then performs the calculation.

# General Purpose Registers (GPR)

General Purpose Registers are registers used to store data temporarily during program execution.
They can be used for different types of tasks like arithmetic, data movement, and logical operations.

- Total Size:

Each general purpose register is:

16 bits = 2 Bytes

And each 16-bit register is divided into:

High part (H) = 1 Byte (8 bits)
Low part (L) = 1 Byte (8 bits)

So:

1 Byte + 1 Byte = 2 Bytes (16 bits)

There are 4 main general purpose registers:

| Register | Purpose | Size | High Byte | Low Byte | Example |
| --- | --- | --- | --- | --- | --- |
| AX – Accumulator Register | • Used in arithmetic operations (ADD, SUB, MUL, DIV) • Main register for calculations | 2 Bytes | AH = 1 Byte | AL = 1 Byte | MOV AL, 05H<br>MOV AH, 02H<br>ADD AL, 03H |
| BX – Base Register | • Used to store base address • Used in memory addressing | 2 Bytes | BH = 1 Byte | BL = 1 Byte | MOV BL, 10H |
| CX – Count Register | • Used as counter in loops • Used in shift and rotate operations | 2 Bytes | CH = 1 Byte | CL = 1 Byte | MOV CL, 05H |
| DX – Data Register | • Used in multiplication and division • Used in input/output operations | 2 Bytes | DH = 1 Byte | DL = 1 Byte | MOV DL, 09H |

# Special Purpose Registers

These registers have specific tasks in CPU operation, unlike general-purpose registers which are used for multiple tasks.

### Segment Registers

- Purpose: Hold the base addresses of memory segments. They divide memory into code, data, stack, and extra segments.

- Size: 16 bits = 2 Bytes

| Register | Segment Type | Purpose |
| --- | --- | --- |
| CS | Code Segment | Holds the starting address of the code being executed |
| DS | Data Segment | Holds the starting address of data |
| SS | Stack Segment | Holds the starting address of the stack |
| ES | Extra Segment | Used for extra data storage, especially in string operations |

Example:

MOV AX, 2000H

MOV DS, AX   ; Load Data Segment register with 2000H

### Pointer Registers

- Purpose: Hold memory addresses (usually of stack or data).

- IP is also called Control / Special Registers (Internal).

- Size: 16 bits = 2 Bytes

- Types:

| Register | Purpose |
| --- | --- |
| SP (Stack Pointer) | Points to top of stack |
| BP (Base Pointer) | Points to base of stack or function parameters |
| IP (Instruction Pointer) | Holds address of next instruction to execute |

Example:

MOV SP, 1000H  ; Stack pointer points to memory 1000H

### Index Registers

- Purpose: Used for indexing memory locations, useful in loops and arrays.

- Size: 16 bits = 2 Bytes

- Types:

| Register | Purpose |
| --- | --- |
| SI (Source Index) | Holds source address for string/data operations |
| DI (Destination Index) | Holds destination address for string/data operations |

Example:

MOV SI, 2000H  ; Source index points to 2000H

MOV DI, 3000H  ; Destination index points to 3000H

### Flag Register

- Purpose: called Control / Special Registers (Internal)

Indicates CPU status after an operation. It is 1 register that holds multiple flags (bits) .

- Size: 16 bits = 2 Bytes

- Types of Flags:

| Flag Name | Full Form | Purpose | When It Becomes 1 (Set) |
| --- | --- | --- | --- |
| CF | Carry Flag | Indicates carry or borrow in arithmetic | When carry occurs in addition or borrow in subtraction |
| ZF | Zero Flag | Shows result is zero | When result = 0 |
| SF | Sign Flag | Shows sign of result | When result is negative |
| OF | Overflow Flag | Indicates arithmetic overflow | When signed overflow occurs |
| PF | Parity Flag | Checks even parity | When result has even number of 1 bits |
| AF | Auxiliary Carry Flag | Used in BCD arithmetic | When carry occurs from bit 3 to bit 4 |
| DF | Direction Flag | Controls string instruction direction DF=0 → increment index DF=1 → decrement index | When STD sets it or CLD clears it |
| IF | Interrupt Flag | Enables or disables CPU interrupts | When STI sets it (interrupts enabled) or CLI clears it (interrupts disabled) |

Example

1.MOV AL, 05H
SUB AL, 05H   ; Result = 0 → ZF = 1

2.MOV AL, FFH
ADD AL, 01H   ; Carry occurs → CF = 1